[image: image3.png]Agilent EducatorsCorner.com Experiments
e ———————————————— —





[image: image3.png]​

Experiment 6:
 Transistor Inverters

By: Moe Wasserman


College of Engineering


Boston University


Boston, Massachusetts

Purpose:
To determine the voltage range over which a transistor acts as a linear amplifier, and to measure the voltage gain.

Method:

Voltage-transfer plots (vOUT vs vIN) for BJT and MOSFET inverters will be produced, and the dependence of the curve shapes on component values and input conditions will be determined.  The input voltage to the base or gate will be incremented at a fixed VCC or VDD, and the corresponding vCE or vDS will be measured. Observed voltage gains will be compared with calculated values.

A simple gain expression for the BJT is obtained by equating the transistor current with the current through RC.




    for vIN > Vf


(6-1).

Therefore








(6-2),

and


voltage gain = 

.




(6-3)

Doing the same for the MOSFET in  the constant-current region gives




     for vIN > VTR


(6-4),

which leads to


voltage gain = 






(6-5)  

Hardware Setup:

The circuits for the BJT and the MOSFET are shown below.  As in Experiment 4, the 

100 kresistor need not be removed for the MOSFET.  Use transistors for which you measured the i-v characteristics in Experiment 4, and begin with RC  (RD) = 10 kand VCC (VDD) around 15 V.

[image: image1.wmf]
Software Setup:

Following the customary procedure, create instrument panels for configuring the function generator and multimeter for DC operation, and component drivers for all dynamic interactions with the instruments.  A direct I/O driver will be needed for the triple power supply.

Use a knob or slider to set the +25 V power supply to the desired VCC value, and a For Range object to set the range of input voltages, using the offset feature of the function generator.  The offset output from the function generator (or directly from the input) provides the X input to an XY plotter, and the multimeter output provides the Y input.

Procedure:

Before running the experiments, predict the range of vIN values over which the BJT and MOSFET will remain in the forward active or constant-current region, respectively.  Then run the program for each, recording the approximate values for Vf and VTR, and measuring the voltage gains with the use of the markers on the vOUT vs vIN plots.  You must use a small enough increment in vIN to obtain meaningful readings from the curves.

Repeat the experiments with a different value of VCC or VDD.  Should the gains change?  Do they?  Now run the experiments with  a 1 kpullup resistor.  Do the gains change as you would expect?

[image: image2.png]MEIE

Start

From
Thru
Step





Fig. 6-6  Agilent VEE Setup

� EMBED Word.Picture.8  ���











[image: image4.wmf]


_960275177

_960275178

_1030875879.doc
[image: image1.png]Agilent Technologies
EducatorsCorner.com Experiments








_960275175

_960275176

_960275174

