[image: image3.png]Agilent EducatorsCorner.com Experiments
e ———————————————— —





[image: image3.png]​

Capturing a Burst of 91 Low Duty Cycle Pulses

By:  Agilent Technologies 

Purpose:

In this hands-on exercise, you will learn how to use Sequential Single-shot to capture a burst of low duty cycle events with high resolution.

Equipment: 

· Agilent 54520-Series Oscilloscope 

· Agilent 54720-66506 Application Training Board 

1. Using a 50 (Ohm) BNC cable, connect J8 of the Agilent 54720-66506 training board to CH1 of the Agilent 54542A scope.


a. "Close" switch #1 on S1.


b. Set VAR LFCK fully CCW
2. Load the scope setup from the disk file.


a. Press the [blue shift key] and then press [Disk].


b. Select the load scope soft key and then select SET.


c. Turn the general entry knob to select setup from file [LAB3A1.SET].

d. Press execute.

[image: image1.png]runving-avaiting igger

oo s o000
00 mae




3. Press the SINGLE-SHOT/GLITCH button on the training board.

4. Select the HORIZONTAL setup menu and make note of the present sample rate.______ Notice that by using peak detect at this timebase and memory setting (16K on screen), the scope has captured just one point on each of the 91 pulses. Is this sufficient data for detail pulse analysis on each pulse? 

5. Change the timebase to 1 ms/div and then press the SINGLE-SHOT/GLITCH button once again to observe the low duty cycle nature of this pulse stream.

6. Load disk setup file [LAB3A2.SET] as described in step #2.

7. Select the HORIZONTAL setup menu and set sequential to on. The oscilloscope is now setup to capture 91 successive pulses at 50 MSa/s with a sample density of 500 points on each pulse.

[image: image2.png]stopped

T
vt
(12765 62509





8. Press the scope's [Run] key and then press the SINGLE-SHOT/GLITCH button on the training board. 

9. Select the [Display] menu, press the segment # softkey, and then rotate the general entry knob to observe all 91 pulses individually and as a composite.

10. What is the precise time from the initial trigger (first pulse) to the 87th pulse?

 (Hint: The time tag of each segment is displayed below the waveform display area.) 

� EMBED Word.Picture.8  ���











[image: image4.wmf]


_1030875879.doc
[image: image1.png]Agilent Technologies
EducatorsCorner.com Experiments








