Lab 1 – HP VEE Basics

By: Mark Horenstein

Associate Professor
	Dept. of Electrical and Computer Engineering

Boston University
	Boston, Massachusetts
	USA

Objective

1.  Learn the function of the following objects in HP-VEE:
	•	 Formula

•	 Integer and Real Constant

•	 Integer and Real Slider

•	 Alphanumeric Display

•	 Logging Alphanumeric Display

•	 Strip Chart Recorder

•	 Count for Range

•	 Start Button

2. Learn to perform the following tasks in HP-VEE

•	Connect data threads between objects

•	Connect sequence threads between objects

•	Change the title bar of an object

•	Add data inputs to an object

•	Autoscale a display

•	Set a slider to Auto Execute

Equipment

•  HP VEE software package

Experiment
Create a graphical program that computes the following formula:

y(x) = a1 sin x + a3 sin 3x + a5 sin 5x + a7 sin 7x

Display y over the range 0 < x < 360 for various values of a1 ... a7. Your program should include a method for setting these constants and observing the resulting function y(x).

For what values of the four constants a1 ... a7 does y(x) look most like the following square function?

3y

A------+         U-----+        U------+

3        3         3       3        3        3

3        3         3       3        3        3

--+-------+--- ----+------+-------+-------+-------> t

3        3         3       3        3        3

3        3         3       3        3

3       A--------U      A-------U

3