Experiment 4 — Series Circuits

EL 111 - DC Fundamentals

By:  Walter Banzhaf, E.K. Smith, and Winfield Young

University of Hartford

Ward College of Technology

Objectives:

- For the student to investigate the characteristics of a series circuit.

- For the student to verify experimentally, using measured and calculated values, the following series circuit rules:

a. Total circuit resistance equals the sum of the individual resistances.

b. The current is the same at all points in a series circuit.

c. The sum of the voltage drops equals the source voltage.

- For the student to determine the effects of changing a resistor value, upon total resistance, current and distribution of voltage drops.

Equipment:

- Meters:	Agilent 34401A Digital Multimeter (DMM);

Milliammeter or Handheld MM such as the Agilent 971A

  - Power Supply: 	Agilent E3631A DC power supply (0 to 20.0V DC)

  - Resistors:	1 - 3.3 k, 1 - 2.2 k, 2 - 1.2 k

  - Misc:	Component board

Information:

- NEVER use an ohmmeter on a live circuit. Voltage from a circuit can damage an ohmmeter.

- Always use the measured value of resistance for all calculations.

- Always adjust the power supply voltage with the circuit connected.

- When measuring voltage, the voltmeter must be connected across the circuit.

- When measuring current, you must break the circuit and the current meter must be inserted into the circuit (in series).

Procedure:

PART ONE : Total resistance in a series circuit.

1.	Measure each individual resistor and record below.

R1=

R2=

R3=

2.	Connect the circuit in Figure 1. Note that there is no applied voltage.

3.	Measure the total resistance.

Rtotal =

4.	Add the measured values of R1, R2, and R3 recorded in step 1 together and record.

R1 + R2 + R3 =

5.	What conclusions can be made from the results of the previous procedures?

PART TWO: Current relationship in a series circuit.

    - Connect the circuit in Figure 2. Note that this circuit is the same as in Figure 1, except that there is now a DC supply voltage connected. Set the voltage source to 18.0 volts.

Figure 2

    - Break the circuit at point a. Insert the milliammeter or handheld and measure and record the current flow through point a.

Ia =

    - Break the circuit at point b. Insert the milliammeter or handheld and measure and record the current flow through point b.

Ib =

    - Break the circuit at point c. Insert the milliammeter or handheld and measure and record the current flow through point c.

Ic =

    - Break the circuit at point d. Insert the milliammeter or handheld and measure and record the current flow through point d.

Id =

6.	What conclusions can be made from the results of the previous procedures?

PART THREE: Voltage relationships in a series circuit.

1.	Connect the circuit of Figure 3. Set the DC supply voltage to 15.0 volts.

- Measure the voltage drop across each resistor. Record.

V1 =

V2 =

V3 =

Figure 3

3.	Add the voltage drops together and record.

Total of voltage drops =

4.	What conclusions can be made from the results of the previous procedures?

PART FOUR: Effects of changing a resistance size in a series circuit.

- Connect the circuit in Figure 4. Set the DC supply voltage to 12.0 volts. Maintain this value even when circuit changes are made.

2.	Measure the circuit current and record.

I =

3.	Measure the voltage across each resistor and record.

V1 =

V2 =

V3 =

4.	Disconnect the source voltage and measure the total resistance. Record it.

Rt =

5.	Change the size of R1 to 1.2 k. Leave R2 and R3 as they are in Figure 4.

6.	Repeat procedures 2, 3 and 4 and record the new values below.

I =

V1 =

V2 =

V3 =

Rt =

7.	Noting that R1 has been decreased in value, in step 5, answer the following questions.

    - What has happened to the total circuit resistance?

Why?

    - What has happened to the circuit current?

Why?

    - What has happened to the voltage drops across R2 and R3?

Why?

    - What has happened to the voltage drop across R1?

      - Notice that you have been given far more room to answer the question below. This is because this question NEEDS a lot more thought, and explanation.

Why?