Experiment 10 — Superposition

EL 111 - DC Fundamentals

By:  Walter Banzhaf, E.K. Smith, and Winfield Young

University of Hartford

Ward College of Technology

Objectives:

- For the student to calculate circuit voltages and currents, when more than one voltage source is present, by the process of superposition and verify calculations by measurements.

Equipment and parts:

Meters:	Digital Multimeter (DMM); 
Milliammeter or Handheld MM such as the Agilent 971A

Power Supply:	Agilent E3631A Triple Output DC Power Supply (0-20 VDC, +5VDC)

Resistors:	1 - 3 k, 1 - 1.2 k, 1 - 1.6 k

Misc:		Component Board

Information:

Superposition is a process for calculating currents and/or voltages for a component in a circuit which has more than one source.

The superposition techniques involve the following steps:

Step 1	Remove all sources except one. You elect which one remains since eventually each existing source will be the stand-alone source before the analysis is complete. Replace the removed sources with their internal resistances. (Note: for this lab you will use voltage sources with a resistance of zero and will be instructed to replace the source with a short). Calculate the current(s) and/or voltage(s) with the one remaining source in the circuit for the resistor(s) in which you have an interest. Record the amount and direction of current and/or the magnitude and polarity of voltage across each resistor of interest.

Step 2	Remove the source used in Step 1 and replace another source previously removed. Calculate the current(s) and/or voltage(s) of interest, recording directions, polarities and magnitudes of the current and voltage of interest.

Step 3	Repeat Step 2 until all sources in the original circuit have been used.

Step 4	The actual current and/or voltage for any one resistor will be the algebraic sum of the currents and/or voltages found above for that particular resistor.

Procedure:

- Refer to Figure 1. Use the following steps to calculate the current through and the voltage across RL (the resistor of interest) by superposition techniques.

Step 1	Ea is present, Eb is removed. Refer to figure 2. Remove Eb, and replace it with a short. (This assume that the voltage source has no internal resistance.) Solve for the magnitude and direction of the current through RL in figure 2. Calculate the voltage drop across RL. Determine the polarity of the voltage drop. Record the current, its direction, the voltage drop and the polarity on figure 2. Show your calculations in space provided.

Calculations (Eb removed):

IRL

ERL

Step 2	Refer to figure 3. Remove the short and replace Eb. Refer to figure 3. Remove Ea and replace it with a short. Solve for the magnitude and direction of the current through RL in figure 3. Calculate the voltage drop across RL. Determine the polarity of the voltage drop. Record the current, its direction, the voltage drop and the polarity on figure 3. Show your calculations in space provided.

Calculations (Ea removed):

IRL

ERL

Step 3	The actual current through and voltage across RL WILL BE the ALGEBRAIC SUM of the results obtained in steps 1 and 2. Do these summations below and record the resultant current and its direction and the voltage drop and its polarity on Figure 1 on page two.

Algebraic addition:	ERL =                   +                     =

IRL =                    +                     =

- Verify the results of the superposition calculations by circuit measurement.

a) Connect the circuit of Figure 1. NOTE: Make sure that the DMM common is at point b.

b) Measure the following:

ERL =                   , and point           is positive in respect to point          .

IRL =                   , current flows from point           to point          .

c) 	If the calculated results are not reasonable close to the measured results, check your calculations and measurements to find the error.

- On a separate sheet of paper, repeat Procedures 1 and 2 except reverse the polarity of Ea. Show all three circuit diagrams and all calculations as contained in Procedures 1 and 2 of this experiment.