[image: image1.wmf]

[image: image2.png]Agilent EducatorsCorner.com Experiments
e ———————————————— —



EL 351 Linear Integrated Circuits Laboratory

INVERTING AND DIFFERENTIAL SUMMING AMPLIFIERS

By:  Walter Banzhaf

University of Hartford

Ward College of Technology

USA

Equipment:

· Agilent 54622A Deep-Memory Oscilloscope

· Agilent E3631A Triple-Output DC power supply

· Agilent 33120A Function Generator

· Agilent 34401A Digital Multimeter

[image: image3.png]5y



I. Test of an Inverting Summing Amplifier

NOTE: In this schematic, the op-amp power supply pins MUST be connected to +/- 15 V to power the op-amp. In addition, the 20k ( potentiometer must be connected as shown, with the wiper ALSO connected to -15 V. This potentiometer is used to null the output voltage, Vo, to 0.00 when input voltages Va, Vb and Vc are quiescent (i.e., connected to 0.00 V, which is ground).

1. Obtain and measure all resistors. Use measured values in all calculations.

2. Determine, BY CALCULATION, the gain from each input to Vo. Write the equation below with specific numerical gains.

Vo = KaVa + KbVb + KcVc
3. Connect the circuit and make all inputs quiescent. Null the output.

4. Measure the DC gain from each input separately to the output; be sure to keep other inputs quiescent. Compare measured gains with calculated gains.

5. Apply the following inputs simultaneously:


Va = + 5 VDC
Vb = ‑ 10 VDC
Vc = + 3 VDC 


a. predict Vo, using the measured gains from 4. above.


b. measure Vo. Compare predicted Vo with measured Vo.


c. measure Ia, Ib, Ib, and Io in a way that minimally disturbs the circuit. Discuss how you did this, and what the results were. Hint: Kirchhoff’s laws are always true, Ohm’s law works.
II. Design and Test of a Differential Amplifier


1.
Design an amplifier such that:  Vo = 4 Va ‑ 5 Vb ‑ 8 Vc
2. Test the circuit to ensure that it performs properly, in two ways: 

Be sure to measure each gain individually, and also subject the amplifier to three simultaneous inputs. 

Choose input voltages for Va, Vb and Vc that will keep the amplifier in its linear operating range.

� EMBED Word.Picture.8  ���











_1030875879.doc
[image: image1.png]Agilent Technologies
EducatorsCorner.com Experiments








